# Week 9: Donkey Car Simulator Introduction

In-Class Activity Guide

Purpose. This activity introduces the simulation environment that we will use as we move toward the final autonomous-driving project. Today, the goal is to understand the simulator and the control loop—not to train an autonomous model.

# Learning Goals

Launch the Donkey Car Simulator and identify the major controls.

Drive manually and observe how steering and throttle affect vehicle behavior.

Explain the observation-action loop used by Gymnasium.

Identify the meaning of camera observations and common simulator information such as speed and cross-track error.

Explain how a trained model will later connect to the simulator.

# How the Simulator Fits the Course

Simulator  ->  Observation  ->  Python Model  ->  Steering / Throttle  ->  Simulator

The final project is a continuous control problem. A model will repeatedly receive information from the simulator, make a decision, send an action back, and then receive the next observation.

## Software Stack

| Donkey Car Simulator | Visual Unity-based driving environment |
| --- | --- |
| gym-donkeycar | Python/Gymnasium interface to the simulator |
| Gymnasium | Provides reset() and step() environment interaction |
| Python / PyTorch | Will later provide the prediction or control model |

# Before Class: Setup Checklist

Download and unzip the current simulator binary from the gym-donkeycar Releases page.

Confirm that the simulator opens on your computer.

Install the Python interface in your course environment.

Bring the simulator path/location to class.

Do not worry if you cannot yet connect a neural network to the simulator; that is not today's objective.

# Install current gym-donkeycar from GitHub
pip install git+https://github.com/tawnkramer/gym-donkeycar

Reference links: https://github.com/tawnkramer/gym-donkeycar  |  https://github.com/tawnkramer/gym-donkeycar/releases

# Activity 1 - Launch and Explore the Simulator

Step 1. Open the Donkey Car Simulator and select a simple track such as a generated or Warren-style track.

Step 2. Drive manually for several minutes. Focus on behavior rather than speed.

## While Driving, Observe

How sensitive is steering near the center position?

How does higher throttle change the difficulty of staying on the road?

What happens when you enter a curve too quickly?

How difficult is it to recover after leaving the ideal path?

Which parts of the track appear easy or difficult for an automated controller?

# Activity 2 - Think Like the Model

Imagine that the vehicle cannot see the entire track. At each moment, it receives only its current observation and must choose an action.

| What the System Receives | What It Must Decide | What Happens Next |
| --- | --- | --- |
| Camera image and/or state information | Steering and throttle | Vehicle moves and produces a new observation |

Observation_t  ->  Action_t  ->  Observation_t+1  ->  Action_t+1  ->  ...

## Why This Is Different from Ordinary Prediction

A normal classification model predicts one observation and stops.

In the simulator, one prediction changes the vehicle's next state.

A small steering error can create a harder situation on the next time step.

Successful autonomous driving therefore requires stability over an entire sequence, not just good average prediction accuracy.

# Activity 3 - Identify Likely Failure Modes

With a partner or small group, identify at least three situations that might cause an autonomous controller to fail. Examples may include sharp curves, excessive speed, poor recovery, unusual camera views, or repeated small steering errors.

| Potential Failure | Why It Is Difficult | Possible Test |
| --- | --- | --- |

# Activity 4 - The Gymnasium Observation-Action Loop

You do not need to memorize this code today. The purpose is to see how Python communicates with the simulator. The current gym-donkeycar interface follows the Gymnasium pattern.

import gymnasium as gym
import gym_donkeycar
import numpy as np
env = gym.make("donkey-generated-track-v0")
obs, info = env.reset()
action = np.array([0.0, 0.5])   # [steering, throttle]
obs, reward, terminated, truncated, info = env.step(action)
env.close()

## How to Read the Code

| env = gym.make(...) | Create/connect to a simulator environment |
| --- | --- |
| obs, info = env.reset() | Start an episode and receive the first observation |
| action = [steering, throttle] | Choose the vehicle-control command |
| env.step(action) | Send the command and advance the simulator |
| obs | New observation, typically the camera image |
| info | Additional vehicle/track information |

## The Action

The action is a two-value NumPy array: [steering, throttle].

Steering near 0 means approximately straight.

Negative and positive steering values turn in opposite directions.

Throttle controls how strongly the vehicle moves forward.

Test small steering/throttle values first.

## What Comes Back from the Simulator?

Observation (`obs`). Typically represents the visual camera observation used by the agent.

Information (`info`). Available keys depend on the environment/version. Common values can include CTE, speed, collision status, and lap-related information.

| Camera observation | What the vehicle 'sees' |
| --- | --- |
| CTE | Distance from the desired track center |
| Speed | Current vehicle speed |
| Hit / collision | Whether the vehicle contacted an obstacle |
| Lap information | Progress or lap timing when available |

## Important for Week 9

You are not expected to build an autonomous controller today. The goal is to understand the simulator interface and continuous feedback loop.

# Activity 5 - Connect Week 9 DNN Concepts to the Simulator

Recall the DNN workflow from the first part of Week 9:

Driving Inputs  ->  DNN  ->  Steering Prediction

The final project extends this into a continuously operating system:

Simulator Observation  ->  Model  ->  Steering / Throttle  ->  New Simulator Observation

## In-Class Questions

Why is validation accuracy alone not enough to prove that a controller will drive successfully?

How could many individually 'correct' predictions still produce unstable driving?

Why might speed make steering errors more severe?

Why is saving and reloading a trained PyTorch model important for simulator use?

What information would you record while testing an autonomous controller?

# Troubleshooting Checklist

| Simulator will not open | Confirm the correct OS build was downloaded and fully extracted. |
| --- | --- |
| macOS/Linux permission issue | The executable may require permission, e.g., chmod +x donkey_sim. |
| Cannot import gym_donkeycar | Install it in the same Python/conda environment used by Jupyter. |
| Environment cannot connect | Confirm simulator, environment name, host/port settings. |
| Car behaves too aggressively | Reduce throttle and test small steering values first. |
| Different info keys | Print info and inspect keys in your installed version. |

# Reflection / Exit Ticket

What was the most difficult part of manually controlling the simulated vehicle?

Identify one situation where a predictive model may struggle even if offline accuracy is high.

What is the difference between one static steering prediction and continuous vehicle control?

What simulator information would be most useful for evaluating safety and robustness?

What should your final-project team test before claiming that its autonomous agent is successful?

# Bridge to Weeks 10-11

Week 10: Computer vision preprocessing and image augmentation.

Week 11: CNN-based steering prediction from road images.

Final Project: Connect perception, prediction, vehicle control, and validation in the simulator.

Today's goal: understand the environment before asking a model to control it.